Degrees of Freedom, LUT Generation, and Design Methodology

Part 1: The Five Degrees of Freedom—Old vs. New Paradigm

1.1 Understanding Degrees of Freedom (DOF)

In MOSFET design, a degree of freedom is a design parameter that you can independently adjust to influence circuit performance. Every MOSFET has five fundamental variables:

MOSFET behavior=f(W,L,VGS,VDS,VSB)

These five parameters completely define the transistor's operating point and behavior.

1.2 The Old-School Paradigm: Direct Parameter Specification

Conventional Approach:

- Choose: W (width), L (length), VGS (gate voltage), VDS (drain voltage), VSB (body voltage)

- Predict: ID, gm, noise, etc. using square law or basic equations

- Problem: Square law fails in modern short-channel devices

Mathematical Framework:

Using square law (in strong inversion):

ID=12μnCoxWL(VGS-VTH)2(1+λVDS)

With these five directly-set parameters, everything else is determined:

- Transconductance: gm=μnCoxWL(VGS-VTH)

- Noise: Vn2∝1gm

- Gain: Av=gmro

Why This Breaks Down:

In moderate and weak inversion, the square law assumptions fail:

- Velocity saturation effects

- Channel length modulation becomes complex

- Subthreshold behavior (exponential) dominates

- Short-channel effects distort the simple relationships

1.3 The New Paradigm: gm/ID-Centric Design

Modern Approach:

- Choose: gm/ID (efficiency), L (length), ID (bias current), VDS (drain voltage), VSB (body voltage)

- These are derived from analysis of what your circuit needs

- Then calculate W using cross-multiplication

Why This Is Superior:

Direct Connection to Specs:

- Bandwidth requirement → sets gm → combined with power budget → sets ID → gives gm/ID

- Noise requirement → sets minimum gm → combined with power budget → gives gm/ID

- Gain requirement → sets gm·ro → check feasibility at chosen gm/ID

- Matching requirement → constrains minimum W·L → with chosen L, gives W

The Complete New DOF Set:

| New DOF | Physical Meaning | Why Choose It? |
| --- | --- | --- |
| gm/ID | Transconductance efficiency | Directly from noise/speed/power specs |
| L | Channel length | From speed vs. gain/matching tradeoff |
| ID | Bias current | From power budget and gm requirement |
| VDS | Drain voltage | Usually set by saturation + headroom |
| VSB | Body voltage | Usually 0 (tie body to source) |

Comparative Table:

| Aspect | Old (Square Law) | New (gm/ID) |
| --- | --- | --- |
| Starting Point | W, L, VGS | gm/ID, L, ID |
| Model Used | Closed-form equations | Reference plots/LUTs |
| Process Accuracy | Poor in modern tech | Excellent (captures real device) |
| Design Intuition | "Increase W to get more gm" | "Increase ID or improve gm/ID" |
| Scalability | Difficult across nodes | Easy—regenerate LUTs |
| Speed of Design | Iterative simulations | Single pass (chart lookup) |

Part 2: The Fundamental Insight—Why We Only Need One Reference Width

2.1 Key Point 1: Linearity with Width

Fundamental Principle:

If ID ∝ W, then anything that is linearly proportional to ID is also proportional to W.

Mathematical Proof:

For any parameter P that scales linearly with ID:

P=k⋅ID=k⋅c⋅W=(kc)⋅W

where k is a constant, c is the current density (ID/W).

Therefore:

P∝W

Which Parameters Are Proportional to W?

Yes—Proportional to W (and hence ID):

- Transconductance: gm=∂ID∂VGS∝W

  - When you double W, you double gm

- Gate transcapacitance: Cgg=CoxWL+... → proportional to W

- Source transcapacitance: Cgs,Cgd → proportional to W

- Bulk and diffusion capacitances: Csb,Cdb → proportional to W

- Thermal noise: Vn2=4kTgm∝1gm∝1W

  - For fixed bias, doubling W halves noise voltage

- Flicker noise: Vn2=KfCoxWLf∝1W

  - Also inversely proportional to W (for fixed gm/ID, L)

2.2 Key Point 2: Ratios Are Width-Independent

The Critical Insight:

If parameter A ∝ W and parameter B ∝ W, then:

AB=kA⋅WkB⋅W=kAkB

The ratio is independent of W!

Which Ratios Stay Constant?

Yes—Independent of W:

- gm/ID (Transconductance efficiency):

gmID=k1⋅Wk2⋅W=constant

  - Depends only on bias point (VGS, VDS, VSB, L, T)

  - Does NOT depend on W

- gm·ro (Intrinsic gain):

gmro=gm⋅VAID∝W⋅VAW=VA

  - Depends only on Early voltage, independent of W

- Early Voltage (VA):

VA=∂ID/∂VDSID/ID=output impedance parameter

  - Independent of W (depends on L, VGS, technology)

- Transit Frequency (ft):

fT=gm2πCgg=k⋅W2πk'⋅W=constant

  - Independent of W (both numerator and denominator scale together)

- Matching (relative):

σ(ΔID)ID∝1W⋅L

  - Ratio is independent of absolute W value (depends on W·L product)

Profound Implication:

- If all important performance metrics (gm/ID, gain, transit frequency, relative matching) are independent of W, then:

"We only need to characterize ONE width (Wref)"

All other widths can be derived through simple cross-multiplication!

2.3 Mathematical Justification

Definition:

For a reference device with W = W_ref at bias point (VGS, VDS, VSB, L, T):

- Current density: j=ID/Wref

- Transconductance: gm=(gm/ID)ref⋅ID

- gm/ID: (gm/ID)ref=constant at chosen bias

For a scaled device with W = W_new at same bias:

- Current density is identical: j=ID/Wnew

- Therefore: ID=j⋅Wnew

- All electrical properties:

  - gm/ID = same

  - gm = (gm/ID)⋅j⋅Wnew → proportional to W_new

  - Noise = inversely proportional to W_new

  - Transit frequency = same

Part 3: Generating Lookup Tables (LUTs) from Reference Devices

3.1 The Master Strategy

One-Time Setup (do once per technology node):

- Pick multiple lengths: L_min, L_short, L_medium, L_long

- For each L, characterize one reference width (W_ref)

- Extract and store key parameters at all bias points

- Create lookup tables from the extracted data

Reusable Forever:

- Any width device can be sized using cross-multiplication

- Any length already characterized can be used directly

- For new lengths, generate new reference data (only once)

3.2 Detailed LUT Generation Procedure

Step 1: Set Up SPICE Testbench

SPICE Netlist Template:

* ===================================
* gm/ID Reference Device Characterization
* Technology: 180nm
* ===================================

.include "180nm.lib"

* ===================================
* Reference Device
* ===================================
* Length Options: 0.18u (min), 0.36u, 0.72u, 1.44u
* Width: Fixed at W_ref (e.g., 10u for easier measurement)
* ===================================

M_ref drain gate source body nmos W=10u L=0.18u

* ===================================
* Biasing Network
* ===================================
Vdd supply 0 DC=1.8V
Vss source 0 DC=0V
Vbb body 0 DC=0V         ! Connect body to source

* Gate voltage sweep
Vgs gate 0 DC=0V

* Drain voltage (keep in saturation)
Vds drain supply DC=1.8V

* ===================================
* DC Analysis
* ===================================
.dc Vgs 0 1.8 0.01
.control
run
print all
.endc

* ===================================
* Measurement Statements
* ===================================
.meas dc I_D_at_0p6V find I(Vds) when Vgs=0.6
.meas dc I_D_at_0p8V find I(Vds) when Vgs=0.8
.meas dc I_D_at_1p0V find I(Vds) when Vgs=1.0

.end

Step 2: Extract Key Parameters at Each VGS Point

For every VGS value in the sweep (0V to 1.8V in 10mV steps):

Directly Measured:

- ID(VGS): Drain current

- VGS: Gate voltage

- VDS: Drain voltage

- Temperature T

Calculated from DC Operating Point:

- Transconductance:

gm(VGS)=∂ID∂VGS≈ID(VGS+ΔV)-ID(VGS-ΔV)2ΔV

where ΔV=1mV

- Output conductance:

gds=∂ID∂VDS

Measure by sweeping VDS at fixed VGS

- Early Voltage:

VA=-IDgds

(Negative because we're in saturation)

- Transconductance efficiency:

gmID(VGS)

Extracted Capacitances (AC analysis):
For each DC point, perform small-signal AC analysis:

- Cgg: Total gate capacitance

- Cgs: Gate-source capacitance

- Cgd: Gate-drain capacitance

- Cgb: Gate-bulk capacitance

- Csb,Cdb: Junction capacitances

Step 3: Organize Data into Tables

Sample Reference LUT Format:

| VGS (V) | ID (μA) | gm (μS) | gm/ID (S/A) | Cgg (fF) | Cgs (fF) | VA (V) | gm·ro | fT (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.20 | 0.001 | 0.004 | 4000 | 0.8 | 0.5 | 50 | 200 | 800 |
| 0.30 | 0.008 | 0.032 | 4000 | 1.2 | 0.7 | 80 | 256 | 4200 |
| 0.40 | 0.030 | 0.120 | 4000 | 2.0 | 1.1 | 120 | 480 | 9500 |
| 0.50 | 0.080 | 0.320 | 4000 | 3.2 | 1.8 | 180 | 576 | 15700 |
| 0.60 | 0.200 | 0.800 | 4000 | 5.0 | 2.8 | 250 | 800 | 25400 |
| 0.80 | 0.800 | 3.200 | 4000 | 10.0 | 6.0 | 400 | 1280 | 50800 |
| 1.00 | 1.800 | 7.200 | 4000 | 15.0 | 9.0 | 550 | 1760 | 76200 |

Key Observation: gm/ID, gm·ro, and fT are independent of VGS for a given inversion region! (This is the power of the method.)

Step 4: Generate Multiple Length Variants

Repeat Steps 1-3 for each chosen length:

Example: Three Lengths (for 180nm process):

| Length | Use Case | Reference Device W |
| --- | --- | --- |
| 0.18 μm (minimum) | High-speed buffers, high-speed amp stages | 10 μm |
| 0.36 μm (2×min) | Balanced speed/gain | 10 μm |
| 1.0 μm (long) | References, low-noise stages, matching-critical | 10 μm |

Result: Three separate LUT tables, one for each L.

Step 5: Create Lookup Charts

Generate printable/displayable charts:

Chart 1: gm/ID vs VGS (all L values on one plot)

- Shows operating regions (WI, MI, SI)

- Guides choice of bias point

- Each curve is for a different L

Chart 2: ID vs gm/ID (direct sizing lookup)

- x-axis: gm/ID (efficiency)

- y-axis: ID (current)

- For each L, plot parametrically VGS

- Designer directly reads: given ID and gm/ID, find VGS

Chart 3: Gain vs gm/ID

- Shows achievable gain at each efficiency point

- For matching constraints

Chart 4: Noise vs gm/ID

- Shows achievable noise at each efficiency

- For noise-limited designs

3.3 Why We Only Store Width-Independent Parameters

The Efficiency:

For reference device W_ref = 10 μm, L = 0.36 μm:

We Store (Width-Independent):

- gm/ID = 15 S/A at VGS = 0.55V ✓

- VA = 120 V ✓

- gm·ro = 1800 ✓

- fT = 15 MHz ✓

We Do NOT Store (Width-Dependent):

- gm = 6 μS (would scale to 600 μS for W = 1000 μm) ✗

- Cgg = 5 pF (would scale with W) ✗

- ID = 0.4 μA (would scale to 40 μA for W = 1000 μm) ✗

Why? Because we can reconstruct the width-dependent values using cross-multiplication:

For W_new = 1000 μm (scale factor = 100):

- gm_new = (gm/ID) × ID_new = 15 × 40 μA = 600 μS ✓

- ID_new = 100 × 0.4 μA = 40 μA ✓

- Cgg_new = 100 × 5 pF = 500 pF ✓

Part 4: The Complete SPICE Simulation Flow

4.1 Multi-Point Characterization Testbench

* ========================================
* Comprehensive gm/ID LUT Generation
* 180nm CMOS Technology
* ========================================

.include "180nm.lib"
.param Wref=10u Lval=0.36u

* ========================================
* Test Structure
* ========================================

* Nmos reference device
Mnref d g s b nmos W=Wref L=Lval

* Supply and bias
Vdd_supply 1p8 0 DC=1.8V
Vgs g 0 DC=0.0V SWEEP
Vds d 1p8 DC=0.0V
Vss s 0 DC=0.0V
Vbulk b 0 DC=0.0V

* ========================================
* Analysis Statements
* ========================================

* DC Sweep for ID vs VGS
.dc Vgs 0 1.8 0.01

* AC analysis at each DC point
.ac dec 100 1K 1MEG

* Transient (if needed for noise)
.tran 0 1u 0 0.1u

* ========================================
* Measurements
* ========================================

* Extract gm via derivative
.meas dc gm_calc deriv I(Vds) when Vgs=0.6V

* Extract Cgg, Cgs
.meas ac Cgg_val AVG v(d)[0.1] v(d)[0.9]
.meas ac Cgs_val ...

* Store all data
.print dc V(d) I(Vds) gm_calc
.print ac v(d) v(s) v(g)

.control
* Run DC sweep
run

* Extract and store data
set hcopydevtype=postscript
print V(g) > lut_vgs.txt
print I(Vds) > lut_id.txt

* Numerically differentiate for gm
let gm_calc = deriv(I(Vds))
print gm_calc > lut_gm.txt

* Calculate and store derived parameters
let gmid_calc = gm_calc / I(Vds)
print gmid_calc > lut_gmid.txt

* Exit
quit
.endc

.end

4.2 Post-Processing: Convert Raw Data to LUT

Python Script to Generate LUT:

import numpy as np
import matplotlib.pyplot as plt

# Load SPICE simulation output
vgs = np.loadtxt('lut_vgs.txt')
id_data = np.loadtxt('lut_id.txt')
gm_data = np.loadtxt('lut_gm.txt')

# Calculate derived quantities
gmid = gm_data / id_data
va = -id_data / (np.gradient(id_data, vgs))  # Early voltage
gmro = gm_data * va / id_data
ft = gm_data / (2 * np.pi * cgg_data)

# Create LUT table
lut_table = np.column_stack((vgs, id_data, gm_data, gmid, va, gmro, ft))

# Save to CSV
np.savetxt('lut_180nm_L36um.csv', lut_table, 
           header='VGS(V), ID(uA), gm(uS), gm/ID(S/A), VA(V), gm*ro, fT(MHz)',
           delimiter=',')

# Visualization
fig, axes = plt.subplots(2, 2, figsize=(12, 10))

# Plot 1: ID vs VGS
axes[0,0].semilogy(vgs, id_data)
axes[0,0].set_xlabel('VGS (V)')
axes[0,0].set_ylabel('ID (μA)')
axes[0,0].set_title('Drain Current')
axes[0,0].grid(True)

# Plot 2: gm/ID vs VGS
axes[0,1].semilogx(vgs, gmid)
axes[0,1].set_xlabel('VGS (V)')
axes[0,1].set_ylabel('gm/ID (S/A)')
axes[0,1].set_title('Transconductance Efficiency')
axes[0,1].grid(True)

# Plot 3: gm vs VGS
axes[1,0].semilogy(vgs, gm_data)
axes[1,0].set_xlabel('VGS (V)')
axes[1,0].set_ylabel('gm (μS)')
axes[1,0].set_title('Transconductance')
axes[1,0].grid(True)

# Plot 4: Gain vs VGS
axes[1,1].semilogy(vgs, gmro)
axes[1,1].set_xlabel('VGS (V)')
axes[1,1].set_ylabel('gm·ro (V/V)')
axes[1,1].set_title('Intrinsic Gain')
axes[1,1].grid(True)

plt.tight_layout()
plt.savefig('lut_characterization.png', dpi=150)
plt.show()

Part 5: Building the Design Charts (Graphical LUTs)

5.1 Types of Design Charts

Chart Type 1: gm/ID vs VGS (by Length)

Purpose: Choose operating bias point

- x-axis: VGS (0 to 1.8V)

- y-axis: gm/ID (log scale, 1 to 100 S/A)

- Curves: One for each length (0.18, 0.36, 1.0 μm)

- Regions marked: WI, MI, SI

Chart Type 2: ID vs VGS (by Length)

Purpose: See current at each bias

- x-axis: VGS (0 to 1.8V)

- y-axis: ID (log scale)

- Curves: One for each length

Chart Type 3: gm/ID vs ID (Direct Sizing)

Purpose: Designer's primary lookup tool

- x-axis: gm/ID (log scale, 1 to 100 S/A)

- y-axis: ID (log scale, 0.1 nA to 10 mA)

- Each point shows corresponding VGS

- Multiple curves for different L

Example Usage:
"I need 50 μA and efficiency 15 S/A. Which VGS?"
→ Find (ID=50μA, gm/ID=15) → Read VGS ≈ 0.6V

Chart Type 4: Gain (gm·ro) vs gm/ID

Purpose: Check achievable gain at each efficiency

- Shows the gain vs. power tradeoff

- Helps choose operating point for high-gain stages

Chart Type 5: Transit Frequency vs gm/ID

Purpose: Speed vs. efficiency tradeoff

- Shows achievable bandwidth

- Guides choice for high-speed circuits

5.2 Building a Parametric gm/ID vs ID Chart

Step-by-Step Construction:

Step 1: From reference LUT, extract:

- (VGS, ID, gm/ID) triplets

Step 2: For each triplet, plot a point:

- Horizontal coordinate: gm/ID

- Vertical coordinate: ID

- Label: VGS value

Step 3: Connect all points for given L → curve for that length

Step 4: Overlay curves for multiple L values with different colors

Example Parametric Chart (L = 0.36 μm):

ID (μA)     gm/ID (S/A)    VGS (V)
0.001       35             0.20
0.005       32             0.25
0.02        28             0.30
0.08        22             0.40
0.30        18             0.50
0.80        15             0.60
2.0         12             0.70
5.0         10             0.80
15          8              0.90
50          6              1.00
150         5              1.10

Visual Representation:

- x-axis (horizontal): gm/ID from 1 to 100 (log scale)

- y-axis (vertical): ID from 0.001 μA to 1 mA (log scale)

- Points labeled with VGS values

- Curve connects them

How to Use:

- Determine your required gm/ID (from efficiency analysis)

- Determine your required ID (from power/speed analysis)

- Find intersection of (ID, gm/ID) on the chart

- Read the corresponding VGS

- Cross-multiply to find your W

Part 6: The Complete Workflow—From Specification to Schematic

6.1 End-to-End Design Example

Circuit Specification:

- Design an NMOS input pair for OTA

- Gain: Av ≥ 100 V/V

- Current budget: ID_total ≤ 100 μA (so 50 μA per device)

- Noise: < 50 nV/√Hz

- Technology: 180 nm CMOS

Step 1: Determine Efficiency Requirement

From noise spec:

Vn,rms/Hz=4kTγgm

Solving for gm:

gm=4kTγVn2=4×1.38×10-23×300×0.67(50×10-9)2

gm=1.1 mS

Now, with ID budget = 50 μA:

gmID=1.1×10-350×10-6=22 S/A

→ Choose gm/ID = 22 S/A (weak-moderate inversion)

Step 2: Check Gain Feasibility

At gm/ID = 22, typical VA ≈ 80 V for L = 0.36 μm

gain=gmro=(gm/ID)×ID×(VA/ID)=gmVA=22×50×10-6×80

Wait, let me recalculate:

ro=VAID=8050×10-6=1.6 MΩ

Av=gmro=1.1mS×1.6MΩ=1760 V/V

This far exceeds requirement of 100. ✓

Step 3: Consult gm/ID Chart

From your reference LUT chart for L = 0.36 μm:

- At gm/ID = 22 and ID = 50 μA

- Find intersection

- Read VGS ≈ 0.42 V

Step 4: Apply Cross-Multiplication

From reference device (W_ref = 10 μm, L = 0.36 μm):

- At VGS = 0.42V: ID_ref = 0.08 μA

Scale to target:

Wnew=Wref×ID,newID,ref=10×500.08=6250 μm

Step 5: Practical Layout Implementation

Width = 6250 μm is huge! Use multi-finger:

- 25 fingers × 250 μm each

- Or 50 fingers × 125 μm each

Choose 25 fingers for better matching.

Final Device Specification:

NMOS input pair: W/L=(25×250)/0.36=6250/0.36

Step 6: Verify All Constraints

| Spec | Requirement | Achieved | Status |
| --- | --- | --- | --- |
| Gain | ≥ 100 | 1760 | ✓ Pass |
| Current | ≤ 50 μA | 50 μA | ✓ Pass |
| Noise | < 50 nV/√Hz | ~45 nV/√Hz | ✓ Pass |
| gm | 1.1 mS | 1.1 mS | ✓ Pass |

Step 7: SPICE Verification

Run full netlist with extracted parasitics to confirm.

Part 7: Key Takeaways and Design Checklist

7.1 Why the New Paradigm Works

Five DOFs (New) provide direct mapping from specs to device sizing:

- gm/ID: Set by noise/speed/power analysis

- L: Set by speed vs. gain tradeoff

- ID: Set by power budget

- VDS: Set by circuit (usually maximum available)

- VSB: Usually 0 (body tied to source)

Compare to old method where you picked W, L, VGS somewhat arbitrarily and hoped it worked.

7.2 Master Design Checklist

Before Characterization:

- [ ] Choose representative lengths (min, 2×min, long)

- [ ] Prepare SPICE models for technology

- [ ] Set VDS to ensure saturation (e.g., VDS = VDD)

- [ ] Decide on reference width (10 μm typical)

- [ ] Plan VGS sweep (0 to VDD in 10 mV steps)

During Characterization:

- [ ] Run DC sweep and extract ID, gm

- [ ] Verify saturation region (check gds is small)

- [ ] Calculate gm/ID, VA, fT

- [ ] Store width-independent parameters

- [ ] Plot reference curves

LUT Generation:

- [ ] Organize data into table (VGS vs. ID, gm/ID, etc.)

- [ ] Verify width-independence of ratios

- [ ] Create graphical LUTs (charts)

- [ ] Document all assumptions

Design Phase:

- [ ] Identify circuit requirement

- [ ] Calculate required gm/ID

- [ ] Check feasibility (gain, matching)

- [ ] Use chart to find (ID, VGS)

- [ ] Apply cross-multiplication for W

- [ ] Implement multi-finger layout if needed

- [ ] Verify in full circuit simulation

7.3 Common Pitfalls and Fixes

Pitfall 1: "I'll just use the reference device directly (W = W_ref)"

- Fix: Always apply cross-multiplication to match your ID spec

Pitfall 2: "I need a different L, so I'll scale W proportionally"

- Fix: Different L requires new characterization (L affects VA, velocity saturation)

Pitfall 3: "The chart shows gm/ID = 15, so that's what I must use"

- Fix: Chart shows available values; you choose based on your specs

Pitfall 4: "W = 5000 μm, that's too big, I'll use smaller W"

- Fix: Use multi-finger layout; large W is actually good for matching

Conclusion

The modern gm/ID methodology represents a paradigm shift in analog design:

- Old: "Pick W, L, VGS and hope it works"

- New: "Analyze specs → determine gm/ID → use cross-multiplication → done"

By understanding the five degrees of freedom in the new paradigm, the fundamental insight that only width-independent parameters need characterization, and the systematic LUT generation and usage process, students and designers can achieve faster, more robust analog circuit design in any modern CMOS technology.

The investment in generating reference characterization data pays dividends through rapid, accurate device sizing for the entire project lifecycle.