Comprehensive Guide: gm/ID-Based Device Sizing Using Reference Plots and Cross-Multiplication

Part 1: Foundation and Philosophy

1.1 The Evolution of Analog Design Methodology

Historically, analog designers relied on closed-form equations (like the square law) to size transistors. This worked reasonably well in older, longer-channel technologies where simple models matched reality. However, modern nanoscale CMOS presents challenges:

- Short-channel effects dominate

- Velocity saturation occurs at lower electric fields

- Moderate inversion (MI) is commonly used but lacks simple models

- Process variations are significant

The gm/ID methodology emerged as a solution—a practical, measurement-based approach that:

- Works across all inversion regions (WI, MI, SI)

- Captures real device physics without complex equations

- Provides intuitive design decisions

- Scales across technology nodes

1.2 Why gm/ID Is the Right Design Variable

The transconductance efficiency, defined as:

gmID=signal amplification capabilitypower consumed

is fundamentally important because it directly relates to circuit performance:

For Speed (Bandwidth):

GBW=gm2πCL=(gm/ID)⋅ID2πCL

For Noise:

Vn,in2∝1gm=1(gm/ID)⋅ID

For Gain:

Av=gm⋅ro=(gm/ID)⋅ID⋅ro

Notice that gm/ID appears in every performance metric, making it the natural variable to work with.

Part 2: The Universal Scaling Law—ID ∝ W

2.1 Physical Derivation

Consider the drain current equation in any operating region. At a fundamental level, current density (current per unit width) is determined by:

JD=IDW=f(VGS,VDS,VSB,L,T,process parameters)

The key insight: for fixed biasing (VGS, VDS, VSB), temperature, and length, the current density is constant.

Therefore:

ID=JD⋅W=constant×W

2.2 Why This Holds Universally

In Weak Inversion:

ID=I0WLeVGS/(nUT)1-e-VDS/UT

Current is explicitly proportional to W.

In Strong Inversion (Square Law):

ID=12μnCoxWL(VGS-VTH)2

Again, explicitly proportional to W.

In Moderate Inversion:
Even though there's no simple closed-form equation, the fundamental physics still holds—current flows through the width of the channel.

Physical Analogy:
Think of the transistor channel as a highway:

- Width (W): Number of lanes

- Current (ID): Total traffic flow

- Current density (ID/W): Traffic per lane

Doubling the lanes doubles the traffic capacity, regardless of road conditions (bias, temperature, etc.).

2.3 Experimental Verification

This can be verified experimentally:

- Fabricate devices with W₁, W₂ = 2W₁, W₃ = 3W₁ (same L)

- Bias all at same VGS, VDS, VSB

- Measure: ID₂ = 2×ID₁, ID₃ = 3×ID₁ (within matching limits)

Part 3: Building Reference Plots—The Foundation

3.1 What Is a Reference Device?

A reference device is a carefully chosen transistor with:

Dimensional Specifications:

- Width: W_ref (typically 1 μm, 2 μm, or 10 μm)

- Length: L_ref (typically minimum L, 2×L_min, or longer)

Why These Choices Matter:

- W_ref: Small enough for compact simulation, large enough for good matching

- L_ref: Chosen based on target application:

  - Minimum L: For high-speed circuits

  - 2-3× minimum L: For balanced speed/gain

  - Longer L: For precision analog (ADCs, references, bias circuits)

3.2 Generating Reference Data—Step by Step

Step 1: Set Up SPICE Simulation

Create a testbench with:

* Reference device
M1 drain gate source body nmos W=2u L=0.2u

* DC voltage sources
Vgs gate source DC=sweep
Vds drain source DC=1.2V
Vss source 0 DC=0
Vbb body 0 DC=0

* Measurement statements
.dc Vgs 0 1.5 0.01
.print dc I(Vds) @M1[gm] @M1[gds]

Step 2: Sweep and Extract

Sweep VGS from 0 to VDD (e.g., 0 to 1.5V in 10mV steps) and extract:

- ID: Drain current

- gm = ∂ID/∂VGS: Transconductance

- gds = ∂ID/∂VDS: Output conductance

- Cgg, Cgs, Cgd: Capacitances

Step 3: Calculate Derived Quantities

For each VGS point, compute:

gmID=gmID

gmro=gmgds

fT=gm2πCgg

Step 4: Generate Plots

Create these essential plots:

- ID vs VGS: Shows current-voltage characteristic

- gm/ID vs VGS: Shows efficiency vs bias

- gm/ID vs ID: Direct lookup for sizing

- ft vs gm/ID: Speed vs efficiency tradeoff

- gm·ro vs gm/ID: Intrinsic gain vs efficiency

3.3 Understanding the Reference Plots

Looking at your figures, we see three distinct curves:

Dashed Line (Square Law):

- Theoretical prediction

- Assumes long channel, no short-channel effects

- gm/ID=2VGS-VTH in strong inversion

Blue Curve (Long Channel MOSFET):

- Actual device with larger L (e.g., L = 1 μm)

- Closer to square law but still deviates

- Shows velocity saturation effects

Red Curve (Short Channel MOSFET):

- Modern device (e.g., L = 0.18 μm)

- Significant deviations from square law

- This is what you'll actually design with!

Part 4: The Cross-Multiplication Method—Complete Derivation

4.1 Mathematical Foundation

Starting Point:
We have a reference device characterized by (W_ref, L_ref) with known behavior: ID_ref(VGS) and corresponding gm/ID values.

Design Problem:
We need a new device that delivers ID_new at the same gm/ID as the reference.

Key Assumption:
Since ID ∝ W (proven in Part 2), we can write:

For reference device:

ID,ref=k⋅Wref

For new device:

ID,new=k⋅Wnew

where k is the current density (ID/W), which depends on VGS, VDS, L, etc.

Critical Insight:
If we want the same gm/ID, we must operate at the same VGS, which means same k (current density).

Therefore:

ID,refWref=ID,newWnew=k

Cross-multiplying:

ID,ref⋅Wnew=ID,new⋅Wref

Solving for W_new:

Wnew=Wref×ID,newID,ref

4.2 What Remains Constant?

When using cross-multiplication, the following parameters do not change:

Geometric:

- L_new = L_ref: Length stays the same

Electrical:

- VGS,new = VGS,ref: Gate-source voltage stays the same

- VDS,new ≈ VDS,ref: Should maintain similar drain voltage (both in saturation)

- VSB,new = VSB,ref: Body-source voltage stays the same

Performance Metrics:

- (gm/ID)_new = (gm/ID)_ref: Efficiency stays the same

- Inversion level: WI, MI, or SI stays the same

- Current density (ID/W): Stays constant

4.3 What Scales Proportionally?

Linearly with W:

- ID: Scales by factor (W_new/W_ref)

- gm: Scales by same factor since gm = (gm/ID) × ID

- Capacitances: Cgg, Cgs, Cgd all scale with W

- Matching (absolute): σ(ΔV) ∝ 1/√(WL), so matching improves with larger W

Unchanged:

- Threshold voltage: VTH

- Transconductance efficiency: gm/ID

- Transit frequency: ft = gm/(2πCgg) stays constant (both numerator and denominator scale together)

- Intrinsic gain: gm·ro (if ro doesn't change with W, which is approximately true)

Part 5: Detailed Worked Example from the Figures

5.1 Analyzing Figure 1 (Top Image)

Given Reference Device Specifications:

From the handwritten notes:

- Aspect ratio: WrefLref=2 μm0.2 μm=10

- Reference dimensions: W_ref = 2 μm, L_ref = 0.2 μm

Reading from the Plots:

Looking at the gm/ID vs VGS curve (middle plot):

- Target operating point: gm/ID = 15 S/A (moderate inversion)

- From the red curve (short channel): at gm/ID = 15, we find:

  - VGS ≈ 0.6 V

  - ID_ref ≈ 0.4 μA

Extracting Additional Information:

From ID vs gm/ID plot (bottom):

- Confirms: at gm/ID = 15, ID = 0.4 μA for our reference device

Computing Transconductance:

gm,ref=gmID×ID=15×0.4 μA=6 μS

5.2 Analyzing Figure 2 (Bottom Image)—The Design Problem

New Design Specification:

The problem states we want:

- ID_new = 0.2 mA = 200 μA (much larger current!)

- Same gm/ID = 15 S/A (maintain efficiency)

- Same L = 0.2 μm (use same technology/speed)

Step 1: Identify Known Values

From reference:

- W_ref = 2 μm

- ID_ref = 0.4 μA

- gm/ID = 15

From specification:

- ID_new = 200 μA

- gm/ID = 15 (same efficiency point)

Step 2: Apply Cross-Multiplication Formula

Wnew=Wref×ID,newID,ref

Step 3: Calculate Numerically

Wnew=2 μm×200 μA0.4 μA

Wnew=2 μm×500

Wnew=1000 μm=1 mm

Step 4: Verify Other Parameters

Length remains the same:

Lnew=Lref=0.2 μm

Gate voltage remains the same:

VGS,new=VGS,ref=0.6 V

Transconductance scales linearly:

gm,new=gmID×ID,new=15×200 μA=3 mS

Verification ratio:

gm,newgm,ref=3000 μS6 μS=500=WnewWref ✓

Step 5: Physical Implementation Considerations

A 1 mm wide device is quite large! Practical implementation strategies:

Option 1: Multi-finger Layout

- Break into N fingers: W_finger = W_new/N

- Example: 20 fingers × 50 μm each

- Benefits: Better matching, lower gate resistance

Option 2: Multiple Devices in Parallel

- Use M devices: W_each = W_new/M

- Example: 10 devices × 100 μm each

- Benefits: Modularity, easier layout

Checking Layout Sanity:

Gate resistance:

RG≈13RsheetW2L1Nfingers

For 1 mm device as single finger:

RG≈13×5 Ω/sq×(1000 μm)20.2 μm=8.3 kΩ

With 20 fingers:

RG≈8.3 kΩ20=415 Ω (much better!)

5.3 Complete Design Summary Table

| Parameter | Reference Device | New Device | Scaling Factor |
| --- | --- | --- | --- |
| Width (W) | 2 μm | 1000 μm | 500× |
| Length (L) | 0.2 μm | 0.2 μm | 1× |
| Drain Current (ID) | 0.4 μA | 200 μA | 500× |
| Gate Voltage (VGS) | 0.6 V | 0.6 V | 1× |
| gm/ID | 15 S/A | 15 S/A | 1× |
| Transconductance (gm) | 6 μS | 3 mS | 500× |
| Inversion Region | Moderate | Moderate | — |

Part 6: Complete Design Flow for Students

6.1 Starting from Circuit Specifications

Example: Design an input differential pair for an op-amp

Given Circuit Requirements:

- Input-referred noise: < 100 nV/√Hz at 1 kHz

- Bandwidth: GBW = 10 MHz

- Load capacitance: CL = 5 pF

- Power budget: ID,total < 100 μA per side

- Technology: 180 nm CMOS

Step 1: Determine Required Transconductance

From bandwidth requirement:

GBW=gm2πCL

Solving for gm:

gm=2π×GBW×CL

gm=2π×10×106×5×10-12

gm=314 μS≈300 μS

Step 2: Check Noise Requirement

Input-referred noise (thermal only):

Vn,in2=4kTγgm

For γ≈2/3 (long channel) and T=300K:

Vn,rms=4×1.38×10-23×300×0.67300×10-6×Δf

Vn,rms/Hz≈5 nV/Hz

This is well below 100 nV/√Hz requirement, so noise is satisfied.

Step 3: Choose gm/ID (Operating Point)

Considerations:

- Need moderate power (not ultra-low)

- Want good matching

- Moderate inversion is good balance

Choose: gm/ID = 15 S/A

Step 4: Calculate Required Current

ID=gmgm/ID=300 μS15=20 μA

This is well within the 100 μA budget. ✓

Step 5: Consult Reference Plots

From your 180nm reference plots (assume you generated them):

- At gm/ID = 15, for reference device (W_ref = 2 μm, L_ref = 0.36 μm):

  - ID_ref = 0.8 μA

  - VGS_ref = 0.55 V

Step 6: Apply Cross-Multiplication

Wnew=Wref×ID,newID,ref

Wnew=2 μm×20 μA0.8 μA

Wnew=2×25=50 μm

Step 7: Final Device Dimensions

WL=50 μm0.36 μm

Step 8: Layout Implementation

Use 5 fingers of 10 μm each:

- W_total = 5 × 10 μm = 50 μm

- L = 0.36 μm

- Benefits: Lower R_G, better matching

Step 9: Verification Simulations

Run corner simulations:

- TT corner: Verify ID = 20 μA at VGS = 0.55 V

- FF corner: Check ID doesn't exceed budget

- SS corner: Verify gm still meets spec

- Monte Carlo: Check σ(VTH) and matching

6.2 Decision Tree for Choosing gm/ID

Start: What is your primary constraint?

Path 1: Power is critical (battery-operated, IoT)

- Choose high gm/ID: 20-25 S/A

- Region: Weak to moderate inversion

- Trade-off: Lower speed, higher noise

- Application: Bias circuits, references

Path 2: Speed is critical (high-frequency amplifiers)

- Choose low gm/ID: 5-10 S/A

- Region: Strong inversion

- Trade-off: Higher power, lower efficiency

- Application: Wideband amplifiers, buffers

Path 3: Balanced performance (general purpose)

- Choose medium gm/ID: 12-18 S/A

- Region: Moderate inversion

- Trade-off: Good compromise

- Application: Op-amps, comparators, most analog blocks

Path 4: Noise is critical (sensor front-ends)

- Choose medium-high gm/ID: 15-20 S/A

- Maximize gm for given power

- Also increase W for better matching

- Application: Instrumentation amplifiers, LNAs

Part 7: Advanced Considerations

7.1 When Cross-Multiplication Breaks Down

Limitation 1: Very Large Width Devices

For W > 200 μm in modern processes:

- Gate resistance becomes significant

- Self-heating can change operating point

- Layout parasitics dominate

Solution: Break into multiple parallel devices or fingers.

Limitation 2: Changing Length

Cross-multiplication assumes L stays constant. If you need different L:

- Generate new reference plots for new L

- Cannot simply scale—L affects:

  - Early voltage (VA ∝ L)

  - Velocity saturation

  - Threshold voltage (short-channel effect)

Limitation 3: Different Temperature

Current density changes with temperature:

- Mobility decreases: μ∝T-3/2

- Threshold changes: ∂VTH/∂T≈-2 mV/°C

Solution: Generate reference plots at target temperature.

7.2 Matching Considerations

For current mirrors and differential pairs, matching is critical.

Pelgrom's Law:

σ(ΔVTH)=AVTW⋅L

where A_VT ≈ 5-10 mV·μm for modern processes.

Design for Matching:

For 1 mV matching (3σ):

3σ=1 mV ⟹ σ=0.33 mV

W⋅L=AVTσ2=5 mV⋅μm0.33 mV2≈230 μm2

If you already chose W = 50 μm from cross-multiplication:

Lrequired=23050=4.6 μm

Decision: If speed allows, increase L to 4.6 μm for better matching.

7.3 Multi-Objective Optimization

Real designs involve simultaneous constraints:

Optimization Problem:
Minimize power while meeting:

- Noise < N_spec

- GBW > BW_spec

- Matching < M_spec

- Area < A_spec

Procedure:

- Map constraints to gm/ID space

- Find feasible gm/ID region

- Choose operating point (often middle of feasible region for robustness)

- Use cross-multiplication for W

Part 8: Summary and Design Checklist

8.1 The Power of the Method

The gm/ID cross-multiplication methodology is powerful because:

✓ No complex equations needed
✓ Captures real device physics
✓ Works across all inversion regions
✓ Technology portable (just generate new ref plots)
✓ Fast and intuitive
✓ Directly links specs to geometry

8.2 Complete Design Checklist

Before Design:

- [ ] Generate reference plots for your technology

- [ ] Choose multiple L values (short, medium, long)

- [ ] Include corner simulations (FF, SS, FS, SF)

- [ ] Verify VDS stays in saturation

During Design:

- [ ] Identify circuit specification (gm, ID, noise, etc.)

- [ ] Choose appropriate gm/ID from plots

- [ ] Apply cross-multiplication to find W

- [ ] Check all constraints (power, area, matching)

- [ ] Decide on multi-finger layout if needed

After Design:

- [ ] Simulate full circuit with sized devices

- [ ] Run corner analysis

- [ ] Perform Monte Carlo for matching

- [ ] Extract layout parasitics and re-verify

- [ ] Document assumptions and margins

8.3 Common Mistakes and How to Avoid Them

Mistake 1: "I'll just use the square law to get W, then tweak in simulation"

- Why it fails: Can be off by 2-5× in modern processes

- Better approach: Use gm/ID from the start

Mistake 2: "I need different L, I'll just scale W proportionally"

- Why it fails: L changes current density non-linearly

- Better approach: Generate reference plots for each L of interest

Mistake 3: "My W came out huge (1 mm), I'll just live with it"

- Why it fails: Parasitics, gate resistance kill performance

- Better approach: Use multi-finger or parallel devices

Mistake 4: "The reference plot says ID = 1 μA, so I'll use that"

- Why it fails: That's for the reference device size, not your spec

- Better approach: Always use cross-multiplication to scale

Conclusion

The gm/ID-based cross-multiplication method transforms analog design from an art to a systematic science. By recognizing that ID ∝ W universally, leveraging reference plots that capture real device physics, and applying simple proportional scaling, students can confidently size transistors for any specification.

The method's beauty lies in its simplicity: generate once, use forever (for that technology). Master this technique, and you'll design faster, with better first-pass success, across any CMOS process node.